CO4 AT1

Name: M Deepika

Reg:192311027

FreeRTOS Stack Utilisation and Stack Overflow Analysis

In a FreeRTOS-based motor controller, each task has its own stack that stores local variables, function parameters, return addresses, saved CPU registers, and temporary data created during function calls. Since the five tasks use statically allocated stacks, every task receives a fixed amount of memory at compile or initialization time. The stack cannot automatically grow if a task needs more space.

1. Analysis of the Stack-Utilisation Pattern

The measured stack utilisation of 85–100% is unsafe.

A task using approximately 85% of its stack has only about 15% remaining as safety margin. A task approaching 95–100% has almost no reserve for unexpected execution conditions such as:

- deeper function-call nesting,

- interrupt pre-emption,

- additional local variables,

- library functions such as printf(),

- temporary buffers,

- error-handling routines,

- recursive functions,

- unusual motor-control events occurring simultaneously.

A task showing 100% utilisation should be treated as a probable stack-overflow condition because the actual worst-case execution may already have exceeded the allocated stack before the measurement was taken.

For example, suppose the five tasks show:

| Task | Stack Utilisation | Interpretation |
| --- | --- | --- |
| Motor Control Task | 100% | Critical; likely overflow |
| Sensor Task | 94% | Very high risk |
| Communication Task | 89% | Insufficient margin |
| Display Task | 85% | Borderline |
| Monitoring Task | 62% | Acceptable but should still be tested |

Under high system load, the motor-control, sensor, and communication tasks may execute deeper call chains than during normal operation. This explains why the system resets appear intermittently rather than continuously.

2. How Stack Overflow Corrupts Memory

A FreeRTOS task stack occupies a fixed block of RAM. When the stack exceeds its allocated boundary, new stack data is written outside that block.

Conceptually:

RAM

------------------------------------------------

Task A Stack

------------------------------------------------

Task B Stack

------------------------------------------------

Task B TCB

------------------------------------------------

Task C Stack

------------------------------------------------

Other variables / buffers

------------------------------------------------

Suppose Task A continues pushing data after reaching the end of its stack. The additional values may overwrite Task B's stack, global variables, heap structures, or a Task Control Block (TCB).

The TCB contains important information such as the task's current stack pointer, priority, state information and scheduling data. If stack overflow corrupts these fields, FreeRTOS may attempt to restore an invalid stack pointer or schedule an invalid task.

This can cause:

- corrupted return addresses,

- invalid program-counter values,

- corrupted CPU register restoration,

- HardFault or memory fault exceptions,

- corrupted task priorities,

- scheduler failure,

- watchdog timeout,

- random program execution,

- unexpected MCU resets.

The effect is often delayed. A task may corrupt RAM at one point, but the reset occurs later when another task tries to use the corrupted information. Therefore stack overflow frequently appears as an intermittent or apparently random reset.

3. Stack-Overflow Detection Techniques

Canary Bytes

A known data pattern is placed near the stack boundary, for example:

A5 A5 A5 A5 A5 A5 A5 A5

The system periodically checks whether this pattern is still intact.

If the stack grows into the protected area, the canary value changes and an overflow can be detected.

Advantages: simple, inexpensive, low memory overhead, suitable for MCUs without an MPU.

Limitations: detection occurs only when the canary is checked. Memory may already have been corrupted before detection. A severe overflow may also overwrite regions beyond the canary.

FreeRTOS's software stack checking mechanism follows a similar principle depending on the selected configCHECK_FOR_STACK_OVERFLOW configuration.

MPU Guard Region

Processors containing a Memory Protection Unit (MPU) can reserve a memory region immediately outside the valid task stack and mark it inaccessible.

Valid Task Stack

-------------------------

Guard Region

NO ACCESS

-------------------------

Other RAM

When the stack enters the guard region, the processor immediately generates a memory-protection fault.

Advantages: hardware-based protection, immediate detection, prevents silent corruption of neighbouring memory, highly reliable.

Limitations: requires MPU-capable hardware, uses MPU regions, requires careful memory alignment and configuration.

An MPU guard region therefore provides stronger protection than simple canary checking.

uxTaskGetStackHighWaterMark()

FreeRTOS provides:

uxTaskGetStackHighWaterMark(taskHandle);

The function reports the minimum amount of unused stack space that has existed since the task started.

For example:

UBaseType_t remaining;

remaining = uxTaskGetStackHighWaterMark(xMotorTaskHandle);

If the result is very small, the task has approached its stack limit.

It is useful for profiling and stack sizing, but it should not be considered a complete overflow-protection mechanism.

Advantages: useful for measuring worst observed stack usage, easy to use during testing, helps reduce unnecessarily large stacks.

Limitations: it only reports the lowest free-space level that has been observed. It does not guarantee that an overflow did not occur, and it cannot predict a future execution path that uses more stack.

The three techniques can therefore be summarized as:

| Method | Principle | Detects Before Corruption? | Main Purpose |
| --- | --- | --- | --- |
| Canary bytes | Check known pattern at stack boundary | Usually no | Software overflow detection |
| MPU guard region | Hardware blocks invalid stack access | Yes/very early | Runtime memory protection |
| uxTaskGetStackHighWaterMark() | Measures minimum unused stack | No | Stack profiling and sizing |

4. Systematic Stack-Sizing Methodology

The five stacks should not simply be increased by an arbitrary amount. A systematic process should be followed.

Step 1 – Measure Every Task Separately

Use:

uxTaskGetStackHighWaterMark()

or its task-specific equivalent to determine the minimum unused stack for each task.

Measurements should be recorded over long-duration tests rather than only during startup.

Step 2 – Exercise Worst-Case Conditions

Run the controller under conditions such as:

- maximum motor speed,

- rapid acceleration/deceleration,

- simultaneous communication,

- maximum sensor activity,

- fault handling,

- communication errors,

- repeated task switching,

- maximum interrupt frequency.

This exposes stack paths that may not appear under normal testing.

Step 3 – Identify Stack-Heavy Code

Review tasks for:

- large automatic arrays,

- deeply nested functions,

- recursion,

- large structures passed by value,

- floating-point library routines,

- formatted I/O such as printf().

For example, instead of:

void MotorTask(void *pvParameters)

{

uint8_t buffer[1000];

}

a large buffer may be moved to static/global storage if concurrency and reentrancy requirements permit:

static uint8_t buffer[1000];

This reduces task-stack consumption.

Step 4 – Determine Observed Worst-Case Stack Usage

If a task has an allocated stack of 1000 bytes and its minimum unused space is 100 bytes:

Observed stack used = 1000 - 100

= 900 bytes

Therefore:

Observed utilisation = 900 / 1000 × 100

= 90%

Step 5 – Add Safety Margin

The final stack should include margin above the measured worst-case value.

A practical engineering target is often to leave roughly 20–30% headroom, although the exact margin depends on the safety requirements and variability of the firmware.

For example, if worst-case measured usage is 900 bytes and a 25% margin is selected:

Required stack ≈ 900 × 1.25

≈ 1125 bytes

The stack might therefore be rounded to an appropriate aligned value such as 1152 or 1280 bytes.

Safety-critical tasks such as motor-control and protection tasks should receive additional consideration rather than simply minimizing RAM usage.

Step 6 – Enable Runtime Overflow Detection

During development, enable FreeRTOS stack checking, for example:

#define configCHECK_FOR_STACK_OVERFLOW 2

and provide the corresponding hook:

void vApplicationStackOverflowHook(TaskHandle_t xTask,

char *pcTaskName)

{

taskDISABLE_INTERRUPTS();

while (1)

{

/* Log fault or activate diagnostic LED */

}

}

Where an MPU is available, guard regions should additionally be used.

Step 7 – Repeat Testing

After resizing the stacks, rerun stress tests and recheck the high-water marks.

A suitable target could be approximately:

Normal peak utilisation: < 70–75%

Warning region:          75–85%

Critical region:         > 85%

These are engineering guidelines rather than FreeRTOS requirements. The important factor is maintaining verified margin above the experimentally observed worst case.

Recommended Action for This Motor Controller

Since several task stacks currently reach 85–100% utilisation, the firmware should be considered at high risk of stack overflow.

The immediate corrective actions are:

- Increase the stacks of tasks approaching 100%.

- Enable configCHECK_FOR_STACK_OVERFLOW.

- Implement vApplicationStackOverflowHook().

- Continuously record uxTaskGetStackHighWaterMark() during stress testing.

- Examine large local variables and deeply nested function calls.

- Apply hardware MPU guard regions where supported.

- Perform worst-case motor, communication, interrupt and fault-condition testing.

- Size every task independently using measured worst-case stack consumption plus an engineering safety margin.

Conclusion

The intermittent resets are consistent with insufficient task-stack capacity. When a task exceeds its statically assigned stack, it can overwrite neighbouring stacks, variables or FreeRTOS TCB data, eventually producing scheduler corruption, processor faults or watchdog resets.

uxTaskGetStackHighWaterMark() is mainly a profiling tool, canary checking provides software overflow detection, while MPU guard regions provide the strongest hardware-enforced protection against neighbouring-memory corruption. A robust FreeRTOS design should combine stack profiling, overflow detection, worst-case stress testing and a suitable safety margin when selecting the final stack size.